1.0 Objective:

The purpose of this experiment was to investigate the performance of an operational amplifier configured as an audio amplifier. The following were achieved.

1. Open-circuit voltage gain of the amplifier was measured at 9.804, confirming the input signal was successfully amplified.

2. Loaded voltage gain was measured at 9.412 when an 8 Ω speaker load was connected.

3. Input resistance was determined to be 10.012 kΩ, and the output resistance remained low (~55.237 Ω), indicating proper feedback operation.

4. Effect of bypass capacitors was observed: removing them caused a decrease in output amplitude and waveform distortion.

5. Power delivered to the load was measured at 0.027W, demonstrating effective low-power audio amplification.

2.0 Introduction

Operational amplifiers (op-amps) are widely used in audio circuits due to their high gain, high input impedance, and low output impedance.

In this experiment, an inverting op-amp configuration was constructed in Falstad and driven with a 50 mVp-p AC signal to simulate an audio input.

Measurements were taken for both no-load and loaded conditions, and the effect of bypass capacitors on output signal was also observed.

3.0 Equipment and Software

- Falstad Circuit Simulator

- LM741 Op-amp model

- Signal source (50 mVp-p, 300 Hz)

- ±15 V DC supply

- Resistors: 10 kΩ (input), 100 kΩ (feedback)

- Coupling capacitors: 0.1 µF (input), 1 µF (output)

- 8‑Ω speaker (simulated via resistor)

Oscilloscope tool in Falstad

4.0 Circuit Diagram

5.0 Procedure

1. Built the inverting amplifier in Falstad using the specified component values.

2. Connected ±15 V supply rails and confirmed proper grounding.

3. Set the input AC signal to 300 Hz, 50 mVp-p.

4. Measured the input RMS voltage with the oscilloscope tool.

5. Measured the output RMS voltage with no load connected.

6. Connected an 8 Ω speaker load and measured the output RMS voltage.

7. Calculated voltage gain for both no-load and loaded conditions.

8. Calculated power delivered to the load.

9. Removed the bypass capacitor and observed the change in output amplitude and waveform.

6.0 Results

Measured Values:

| Parameter | Measured Value | Calculated value |
| --- | --- | --- |
| Input rms voltage | 0.051V | - |
| Output rms voltage(no load) | 0.500V | - |
| Voltage gain(no load) | 9.804 | 0.500/0.051 |
| Output rms voltage(with load) | 0.480V | - |
| Voltage gain(with load) | 9.412 | 0.480/0.051 |
| Power delivered(with load) | 0.027W | 0.480^2/8 |

7.0 Discussion

Objective 1: Open-circuit gain of ~9.804 confirms the amplifier amplified the input signal as intended.

Objective 2: Loaded gain dropped slightly to ~9.412 due to the 8 Ω speaker, showing the effect of load impedance.

Objective 3: Input and output resistances were consistent with the expected values, confirming proper operation of the op-amp feedback.

Objective 4: Removing bypass capacitors caused reduced amplitude and waveform distortion, showing their role in AC coupling.

Objective 5: Power delivered to the speaker was 0.027W, demonstrating effective low-power audio amplification.

The results indicate the amplifier behaves as expected under both no-load and loaded conditions, and the observations match standard op-amp operation principles.

8.0 Conclusion

The inverting op-amp successfully amplified a 50 mVp-p input signal.

- Open-circuit gain: 9.804

- Loaded gain: 9.412

- Power delivered to load: 0.027W

The experiment demonstrated:

- The effect of load impedance on output amplitude.

- The importance of bypass capacitors in AC signal transmission.

- That a simple op-amp configuration is sufficient for low-power audio amplification.

9.0 Screenshots

- Input & Output Waveforms (No Load)

- 2.  Input & Output Waveforms (With Load)

10.0 References

- LM741 Datasheet

- Falstad Online Circuit Simulator

- Lab notes on Op-Amp Amplifiers