# DESIGN AND IMPLEMENTATION OF A MULTI-FLOOR BUILDING NETWORK (BLOCK B)

## TITLE PAGE

Project Title: Design and Implementation of a Multi-Floor Building Network Infrastructure (Block B)

COURSE / MODULE: DATA COMMUNICATION AND NETWORKINGCOURSE CODE: COU 07304
INSTRUCTOR: SIR GAUDENCE TESHA

## ABSTRACT

This project presents the design and implementation of a structured network for Block B, a multi-floor academic building. The hierarchical star topology is used, with the core network devices and server located on the first floor for security and efficient management. The network supports both wired and wireless users, ensuring scalability and high performance.

## 1. INTRODUCTION

A computer network is crucial in modern academic institutions for communication, data sharing, and internet access. This project aims to design and implement a structured network for Block B, ensuring security, reliability, and future scalability.

### 1.1 Objectives

- Design an efficient and secure network topology for Block B

- Implement wired and wireless connectivity

- Ensure centralized control and physical security

- Provide detailed budget and resource estimation

## 2. BUILDING AND USER REQUIREMENTS

### 2.1 Building Layout

- Ground Floor: 4 rooms, 2 wired users per room, 1 wireless AP

- First Floor: Server room, core switch, router, firewall

- Second Floor: Classrooms B9, B10, and an office; 2 wired users per room; wireless access for students

- Third Floor: Classrooms B11, B12, B13; 2 wired users per room; wireless access for students

### 2.2 User Requirements

- Internet access for all floors

- File and server access

- Secure wired and wireless connectivity

- High-performance network for classrooms and offices

## 3. NETWORK TOPOLOGY DESIGN

- Topology: Hierarchical Star Topology

- Core layer: Core switch + server + router + firewall on first floor

- Access layer: Access switches and wireless APs on each floor

- Cabling: Cat 6 for backbone and horizontal connections

- Wireless AP placement: Based on user density per floor

## 4. NETWORK DEVICES, SPECIFICATIONS, AND QUANTITIES

| Item | Quantity | Unit Price (TZS) | Total Price (TZS) | Specification |
| --- | --- | --- | --- | --- |
| Core Switch | 1 | 2,000,000 | 2,000,000 | Layer 3, 24–48 Gigabit ports, VLAN, QoS, SFP support |
| Access Switch (24-port PoE) | 4 | 900,000 | 3,600,000 | Layer 2, 24 Gigabit ports, PoE, VLAN support |
| Wireless Access Points | 6 | 350,000 | 2,100,000 | 802.11ac/ax, dual band, PoE, WPA2/WPA3, 50+ users per AP |
| Server | 1 | 3,000,000 | 3,000,000 | Dual Gigabit NIC, static IP, file sharing, authentication, backup services |
| Router / Firewall | 1 | 1,200,000 | 1,200,000 | Gigabit, firewall, routing, secure access |
| Cat 6 UTP Cable | 500 m | 2,000 | 1,000,000 | Horizontal & vertical backbone cabling |
| RJ45 Connectors | 50 pcs | 2,000 | 100,000 | Crimping compatible connectors |
| Patch Panel | 2 | 150,000 | 300,000 | 24-port, Cat6 compatible |
| Network Rack (Cabinet) | 1 | 800,000 | 800,000 | 42U, lockable, for switches & server |
| UPS | 1 | 700,000 | 700,000 | 2kVA, backup for server & core devices |
| Laptop for Configuration | 1 | 1,200,000 | 1,200,000 | Core configuration, software installation |
| Crimping Tool | 1 | 80,000 | 80,000 | Cable termination tool |
| Network Cable Tester | 1 | 150,000 | 150,000 | Cable testing, continuity & speed verification |
| Software (Packet Tracer / network tools) | 1 | 0 | 0 | For simulation and network planning |
| Tool Box & Accessories | 1 | 120,000 | 120,000 | Screwdrivers, pliers, marking tools |
| Transportation & Miscellaneous | 1 | 300,000 | 300,000 | Movement of equipment and installation costs |
| Total Estimated Cost |  |  | 16,500,000 |  |

## 5. IMPLEMENTATION PLAN

### 5.1 Physical Implementation

- Install core and access switches

- Mount wireless APs per classroom/area

- Run Cat6 cabling (backbone & horizontal)

- Secure server room and network rack

### 5.2 Logical Implementation

- IP addressing plan and VLANs per floor

- Wireless SSID and security configuration

- Server service configuration

- Connectivity verification (ping, access, internet)

## 6. PROJECT TIMELINE

| Activity | Duration |
| --- | --- |
| Requirement Analysis | 1 day |
| Network Design & Topology | 1 day |
| Equipment Procurement | 1 day |
| Installation & Setup | 2 days |
| Configuration & Testing | 1 day |
| Documentation & Reporting | 1 day |
| Total | 7 days |

## 7. LABOR / REIMBURSEMENT COST

| Item | Quantity | Unit Price (TZS) | Total (TZS) | Specification |
| --- | --- | --- | --- | --- |
| Network Technicians | 2 persons × 7 days | 250,000 | 3,500,000 | Skilled technicians for installation & configuration |

## 8. CHALLENGES AND SOLUTIONS

- High user density → deploy multiple APs per floor

- Security → server room with lock & firewall

- Cabling complexity → structured cabling with labels & testing

## 9. CONCLUSION

The structured hierarchical star network ensures centralized control, efficient management, scalability, and high-performance connectivity for both wired and wireless users. The design supports current needs and allows for future expansion.

## 10. RECOMMENDATIONS

- Upgrade backbone to Fiber optic in the future

- Implement network monitoring tools

- Add secondary backup internet connection

## 11. REFERENCES

- Lecture Notes on Computer Networks

- IEEE Standards for Networking

- Networking Fundamentals Textbooks